To the PLC maintainers at Daresbury,

This folder contains:

- Our current PLC code (lightly commented)

- A draft version of proposed PLC code for your system

- The spreadsheets we use to generate G-code recipes for each layer

- A diff report comparing our code to a version I received in February from one of your winders

- The Electrocraft motor controller settings for our winding head

### Differences in Winder Configuration

Our winder differs from yours in a few key ways:

- We do not use Safety-Limited Speed (SLS). Instead, we’ve repurposed the SLS switch for a custom speed-regulator (explained below).

- We do not use the wire-break switch in the winding head. Instead, we detect a wire break by monitoring the low end of the load cell range, which we’ve found more reliable. The switch was causing issues with 24v power supply.

### Improved Performance and Safety

Our PLC code enables higher winding speeds while also improving protection against over-tension and axis drive damage.

#### Faster Operation

- The spool motor in the winding head rotates bidirectionally. This allows it to actively unspool wire when needed—crucial at high speeds, where motor drag alone could over-tension the wire.

- When the speed-regulator switch is on, XY movements are dynamically adjusted to prevent the winding head from pulling excessive tension.

- The Z-arm moves at full speed when not under load (i.e. when no wire is attached), and more slowly when carrying tension.

#### Safer Operation

- We calculate a safe maximum XY speed to ensure that neither axis exceeds its rated limits (825 mm/s X, 600 mm/s Y). This prevents over-speed faults and protects the servos.

- We continuously calculate the stopping distance based on current motion parameters. This allows dynamic speed/acceleration changes without risking overshoot.

- We monitor for wire breaks and over-tension, issuing audible alerts and automatically pausing winding when they occur.

In practice, either of the following changes may be sufficient to enable higher-speed XY motion:

- Implementing the XY speed regulator, or

- Enabling bidirectional control on the Electrocraft controller (the simpler option).

### G-code and Calibration Differences

We use slightly different G-code and calibration methods for the U and V layers.

#### Calibration

- We calibrate the APA using 9 points (3 on each side), instead of ~86.

- XML files for U and V layers are generated identically by copying several columns on the last sheet.

#### Winding Behavior

- Acceleration and jerk values are higher across all axes.

- Speed is regulated by the speed-regulator, not hardcoded in the G-code (though this is toggleable).

- Each wire placement (typically at APA corners) is independently adjustable in G-code for better precision and reduced risk of wire misplacement.

- At combs, the winding head makes an extra motion to seat the wire in the first teeth of the comb.

- We eliminate extraneous movements, reducing G-code by ~12 lines per wrap.

#### Spreadsheet Details

The calibration sheets include:

- An "input pins" tab

- An "interpolation" tab, which:

  - Generates test scripts for verifying calibration

  - Allows per-pin adjustments

- An XML output tab, producing final XML files

The winding sheets include:

- Options for pausing at combs and board gaps

- Parameters for adjusting pause height above boards

- Settings for single vs. two-part transfer motions

- Per-point placement adjustments (e.g., Head A-side corner vs. Head B-side corner)

### Adapting Our Code for Your Use

To adapt our code to your system, you might:

- Re-enable Safety-Limited Speed (SLS) in the XY motion routine.

  - This can be done using the speed regulator: set a reduced “regulated speed” when SLS is triggered, except when near the end of a move (to avoid overshoot).

- Reconfigure the Electrocraft controller using the settings provided.

The included _UK.acd file demonstrates one way to implement Safety-Limited Speed (SLS) as a special case of the speed regulator. Based on my understanding of your current implementation, this approach may address a potential issue in which the winder cannot decelerate quickly enough after receiving a slow-down command—resulting in position overshoot and possible collision with the end-of-travel (EOT) stop.

The G-code files should be usable with minimal modification once you're familiar with adjusting wire positions.

To adapt our system to yours, I recommend the following order of implementation:

- Retune the Electrocraft controller – This will immediately enable higher-speed operation.

- Adopt the simplified G-code and calibration approach – These are straightforward to implement and help streamline setup.

- Adopt some or all of our PLC code – This step involves more significant changes to winding behavior and may require further discussion.

Please feel free to reach out with any questions. I'd be happy to schedule a Zoom call to review the details together.

Best regards,
 Ben Oye
 oye@uchicago.edu